Removal and replacement of the radio aerial.

This can be carried out taking shortcuts to this procedure but each part of it is done for a reason.

1 . Jack up rear driver’s side and remove the wheel.

2.  Release the arch liner from the sill to at least halfway back pull back out of the way. Make sure you have sufficient new trim clips to re-install this.

3.  Remove the side air intake grill. Be careful not to break the clips on removal , always best to make sure you have spares before you start the job.

4. Remove the air intake. Should be just two trim clips holding this in but may be more.

5.  Remove the outlet from the drain bag. This is done by twisting and pulling. Removal of the air intake and drain bag outlet will make it much easier to insert the cable bung for the aerial.

6. Remove the door from drivers side rear bin.

7. Remove bin side panel. There are two push fit clips fitted to this panel.

8. Remove the bottom of the bin. It may be beneficial to loosen the bolts holding the passenger side bin door in place as there is a tongue between the bin bottom sections that holds them together.

9. There is a connector on the aerial lead that goes through the body just behind the end of the centre console. Pull the edge of the carpet forward that was held in place by the bin door and this will reveal the connector. The connector will prise up from its locating lug while the unit above is in place. Depress the holding lug and disconnect the aerial.

10. Disconnect power supply from relay.

11. Push cable bung and cables back through the inner wing.

12. Undo the round nut at the top of the aerial. If you use pipe grips to do this make sure you use something to stop the nut getting damaged.

13. From underneath the car look up into the wheel arch and you will see that the aerial unit is held in place by a single nut holding the support bracket to the body. Undo this nut and remove the aerial unit complete.

Re-fitting is pretty much a reversal of this process.

If you are wanting to test an aerial before refitting I found that bolting it to one of the holes that holds the drivers side bin lid in place, the one nearest the wing, was the easiest option. It could be connected to the aerial and power connectors easily and it saved keep putting it up under the wing and taking it out again.

Hope this has been of some help.