Instructions to reset rear caliper, handbrake cable and install pads.

USE AXEL STANDS!!!!

- Make sure car cannot roll (block front wheels).

- Remove rear wheel(s).

- Release handbrake.

- Remove clip from the parking brake crank pin.

- Remove pin from parking break/leave (wiggle the handbrake crank to help).

- Remove U shape retaining clip to free handbrake cable from cable support bracket to free the handbrake.

- Disconnect break line from shock (10mm bolt).

- Remove bottom bolt holding the caliper to troque plate.

- Swing caliper up from the bottom and suspend it up using rope or cord so your hands are free to work.

- Check operation of the handbrake crank. It should be free enough to operate by hand so you can visually see the piston moving in and out.

- At this point you could remove the caliper (if binding), strip it out, clear dirt or rust from piston and cylinder using wet and dry sandpaper (2000 grit) or scotchbrite, I wont cover this in these steps….

- Wind the caliper all the way in and then back out at least 1/2 a turn.

- Install new pads (if required) or wind the piston back out until the grove of the piston slots into the nipple of the back of the pad and slots in snug with the back of the shim (if still fitted).

- Move caliper back down into position and snug against the pads.

- Reinstall bottom caliper bolt removed in step 8 (20n-m, 15ft-lbp).

- Reinstall brake line stay bolt removed in step 7.

- Re-attach hand brake cable via U shape retaining clip back to cable support bracket removed in step 6.

Repeat the above steps on the other side

- Check operation of brake cables (both sides should be unattached at this stage) the should move in and out freely with operation of the leaver. Ask someone to pull it while you check the other side.
If one seems a bit seized, try spraying wd40 inside the cable and continue operation until it’s free.

- Check fluid level, start the engine and depress break pedal multiple times (at least 10).

- Turn off engine.

- Attach the break cable on both sides by re-install the pin and clip removed in steps 4 and 5.

- Pull and release the handbrake leaver.

- Check the handbrake crank. It should be just touching the stopper pin. Adjust this at the handbrake leaver by releasing the 10mm locking nut and slacken/tighten the cable (under the middle console). Relock using the 10mm nut. Leaver movement should happen at the parking brake crank as soon as the parking brake leaver is pulled, there should be no looseness in any of the cables.

- Operate the handbrake multiple times (at least 10).

- Re-fit wheels.

- Check for binding of the handbrake. At this point, the wheels should spin freely without binding and hopefully, the handbrake should lock the wheels and stop you from spinning them.

- Don’t forget to tighten your wheel bolts once the car is back on the ground!

Good luck with your MOT!
Mark Simpson